### 🔧 Tools Required

- Peg spanner RH8051TL with long extension or the compact spanner RH8051-BR so you can use a pneumatic hamer.

- Torque wrench

- Pneumatic wrench (for stubborn rams, see note)

- Suitable supports for rear suspension

### ⚙️ Removal Procedure

- Depressurise the hydraulic system

  - Pump the brake pedal 20–30 times with ignition OFF until both warning lamps glow dimly.

  - Raise the rear of the car; support under chassis rails. Remove rear wheels.

- Access the ram assembly

  - Remove the spring covers.

  - Locate the height control ram inside each rear spring tower.

- Unlock the piston from the isolator cone

  - Insert RH8051TL or RH8051-BR spanner down through the spring and engage the piston slots.

  - Turn clockwise to unlock the piston from the isolator cone.

  - Do not remove completely yet.

- If the piston is seized or difficult to rotate

  - Apply light penetrating oil and allow it to soak.

  - If still stuck, use a pneumatic or electric impact wrench carefully on the spanner with short, low-torque bursts (max 60 Nm) – own risk.

  - Do not apply continuous impact force—this may damage the ram threads or isolator cone seat.

- Disconnect and remove

  - Remove the two top mounting bolts.

  - Disconnect both hydraulic lines, marking upper (bleed) and lower (pressure feed). Cap all openings.

  - Support the suspension arm and lower slowly. Withdraw the ram with the isolator cone attached.

- Separate components

  - Unscrew the piston fully from the cone. Clean and inspect for wear, thread damage, or corrosion.

### 🔩 Refitting

- Refit the isolator cone squarely in the spring pot.

- Screw the piston into the cone by hand, then tighten anti-clockwise with the spanner to 35–40 lbf·ft.

- Reinstall the ram assembly, top bolts, and hydraulic lines (upper = bleed / lower = feed).

- Refit the spring and covers; lower the car.

### 💧 Bleeding

- Refill reservoir with correct fluid RR363

- Loosen ram bleed screw; have assistant depress brake pedal until clean fluid flows.

- Tighten screw and top up reservoir.

### ⚠️ Common Faults & Service Tips

| Symptom | Likely Cause | Remedy |
| --- | --- | --- |
| Height not returning | Height control valve sticking | Flush system, inspect valve spool |
| Fluid leak at top of ram | Upper seal failure | Replace seal kit, inspect bore |
| Persistent pressure loss | Return line leak or air ingress | Check fittings and bleed thoroughly |
| Ram piston seized in cone | Corrosion or hardened fluid residue | Free gently using penetrating oil or impact wrench (low torque) |